Date: Mon, 24 Nov 1997 14:41:39 -0800

From: Stewart D Lyon <slyon@SPACEVECTOR.COM>

To: HEATH@LISTSERV.TEMPE.GOV

Subject: SB100 belts

Hi Gang:

On the subject of belts for the preselector and loading capacitors for the

SB100 (et al), I have found several different ways to go. Although Heath

supplied three belts of the same type for both the preselector and the

loading capacitor, I recommend a  larger belt for the loading cap to lessen

the strain on the front bearing of  the cap.

 First off are o-rings - available from several different manufacturers -

which use a standard "dash number" system to describe the various sizes.

For the preselector use -227 which is 2.11"ID and 0.14" wide and for the

loading cap, use -228, 2.23" ID and the same width. (1/8" width will also

work.)  These belts can be found at appliance repair stores, sometimes ACE

hardware stores, plumbing stores and auto race supply stores. I got mine

free from an aerospace type place so I don't have any price data.

VCR drive belts work.  They are a flat belt (instead of round) and are

dimensioned by circumference.  They are also "stretchier" than o-rings so

they don't give quite the preciseness of control - also they can be shorter.

For the preselector, try a 64 (6.4" circumference, 2.04" ID) and for the

loading a 70 (2.23" ID).  I've seen these belts in radio stores anywhere

from 99 cents to $4.59 ea. If in doubt, use a piece of string or hookup wire

around the pulleys to get the circumference and subtract about 10%.

Phonograph drive belts also work and were what I used in the 70's.  Not many

phonographs around anymore so they may be hard to find.  Try a radio store

that's been in business a long time.

Multi-Volt Murray suggested cutting the length you needed from stock 1/8"

material and gluing the ends together.  I've watched technicians at work do

just that and it looked to me akin to brain surgery and not something I'd try!

BTW - using o-rings for belts was done for years and is not a Heath idea.

The Mil-spec o-ring places have a special mix just for drive belt use.

Hope this helps someone.  Stu - W6CUX

S. D. Lyon, Space Vector Corp.

email: slyon@spacevector.com

Phone: (818)734-2610 or (818)886-6500

FAX:   (818)886-3947

--- --- --- --- --- --- --- --- --- --- --- --- --- --

